City of Bluff City

Specifications For A Zero-Turn Rotary Mower

Bidder must complete and return this form for the bid to be considered responsive. Variances must be clearly identified in the Bidder=s Equipment column (if the bidder=s equipment meets the specifications in the City Specifications column then simply place a  in the Bidder=s Equipment column). Failure to comply with any part of the bid specifications will not remove that bid from consideration but will indicate a variance on which the City alone will determine the importance to the overall performance of the item and suitability for the intended purpose. Equipment bid is to be new, unused, of the Manufacturer=s latest model and delivered to the City site in a ready to use condition. Bid shall be good for 45 days and delivery shall be no later than 14 days following notification of the bid award, unless an extension is specifically authorized by the City Recorder.

City Specifications							Bidder=s Equipment

Brand/Model Proposed (please specify)

Commercial, zero turn, rotary cut mower

Minimum 25 HP Air-cooled engine

(see liquid cooled option below)

Hydro drive system: variable speed control,

10mph forward, 5mph reverse

Adjustable cutting heights from 1" to 6" from

operator=s seat

Minimum 60" width cutting deck, tri-blade, fully

reinforced, floating, adjustable, anti-scalping, minimum

10 gauge top of deck, minimum 7 gauge skirt, and wide

discharge chute

Cast iron pulleys with easily removed tapered hubs

Foot operated deck lift adjustment from operators

seat or electric deck lift adjustment from operators seat

or alternative easy-adjust deck lift (please specify)

Drive tires: 24x12-12, 4 ply turf tread

Padded seat, thick cushion, seat belt

Heavy-duty drive shaft to 90 degree gear box drives

the cutter deck

Dump valves: allows for movement without running

engine

High capacity air filter system

Full pressure oil lubrication system

Parking brake: lever actuated linkage to brakes on both

drive wheel axles

Minimum 10 gallon fuel capacity

Minimum 12 volt battery with alternator

True zero turn radius

Front and rear lights (headlamp and brake)

Electric clutch PTO engagement

Full instrument panel: ammeter, hour meter, key

switch, throttle lever, manual choke, mower

disengagement switch, fuses, safety start module

Safety system: seat actuated engine kill, neutral

interlock, mower engagement BBC switch, parking brake

Operator, parts and service manuals

Warranty: specify manufacturer=s warranty:

Bidder=s delivered price for specifications stated above

Bidder=s price for the same but with a minimum 25 HP liquid cooled engine

Firm Name:

Address/Phone:

Authorized Signature/Date: